4K 3D转换器规格书

2 × 12G-SDI/HDMI2.0 to 12G-SDI/HDMI2.0

| UpDown Converter Series Hardware Specification |  |
| --- | --- |
| 型号 | 2 × 12G-SDI/HDMI2.0 to 12G-SDI/HDMI2.0 |
| 图片 |  |
| 描述 | 2 × 12G-SDI/HDMI2.0 to 2 × 12G-SDI and 2 × HDMI2.0 Up/Down Converter with 3D Support |
| 特征 |  Side by Side, Top and Bottom, Line by Line 3D Mode and Frame Sequential Support  Two 2D Video Convert to One 3D Video Support  One 3D Video Convert to Two 2D Video Support  I ↔ P Conversion ( Built-In Interlacer and DeInterlacer. )  60fps ↔ 59.94fps Conversion  Frame Rate and Format Up/Down Conversion |
| 输入接口 | Video  HDMI2.0×2 ( 2D / 3D )  12G-SDI×2 ( 2D / 3D ) |
| 输出接口 | Converted Video  HDMI2.0×2 ( 2D / 3D )  12G-SDI×2 ( 2D / 3D ) |
| 输入格式 | SDI 12G-SDI 4096x2160p@60/50fps 3840x2160p@60/50fps 6G-SDI 4096x2160p@30/25/24fps 3840x2160p@30/25/24fps 3G-SDI 1920×1080p@60/50fps HD-SDI 1920×1080p@30/25/24fps 1920×1080i@60/50fps (*) 1280×720p@60/50fps HDMI 2.0 4096x2160p@60/50fps 4096x2160p@30/25/24fps 3840x2160p@60/50fps 3840x2160p@30/25/24fps 1920×1080p@60/50fps 1920×1080p@30/25/24fps 1920×1080i@60/50fps (*) 1280×720p@60/50fps *: Interlaced input video is not supported for frame sequential output mode |
| 输出格式 | SDI ( Up/Down Converted ) 12G-SDI 4096x2160p@60/50fps 3840x2160p@60/50fps 6G-SDI 4096x2160p@30/25/24fps 3840x2160p@30/25/24fps 3G-SDI 1920×1080p@60/50fps HD-SDI 1920×1080p@30/25/24fps 1280×720p@60/50fps HDMI 2.0 ( Up/Down Converted ) 4096x2160p@60/50fps 4096x2160p@30/25/24fps 3840x2160p@60/50fps 3840x2160p@30/25/24fps 1920×1080p@60/50fps 1920×1080p@30/25/24fps 1280×720p@60/50fps |
| 音频输入 | SDI : Up to 8 Channels, 24bit, 48KHz HDMI : Up to 8 Channels, 24bit, 192KHz |
| 音频输出 | HDMI ( DVI-D ) /SDI : Up to 8 Channels, 24bit, 48KHz 3.5 mm Phone Jack : 2 Channels, 24bit, 48KHz |
| Plug & Play 即插即用 | ✓ |
| 电源 | 7.5V ~ 15V |
| Dimension 尺寸 | L:171mm×W:103mm×H:35.1mm |

2/2