| Model | 12G-SDI to HDMI2.0-S |
| --- | --- |
| 产品图片 |  |
| 描述 | 12G-SDI/Quad-Link 3G-SDI to HDMI 2.0 Up/Down Converter |
| 特征 | - Quad-Link 2 Sample Interleave ( 2SI ) Supported - Square Division Quad Supported - Quad-Link/Dual-Link/Single Link Supported - 60fps ↔ 59.94fps Conversion - Multi-Standard Support ( 12G/6G/3G/HD-SDI ) - Auto Source Format Detection |
| 输入接口 | Video - 12G-SDI×1 - 6G-SDI×1 - 3G-SDI×2 ( Dual-Link 6G-SDI, Quad-Link 3G-SDI Supported ) |
| 输出接口 | Up/Down Converted Video - HDMI 2.0 Audio - 3.5 mm Phone Jack×1 Loop-Through Video - 12G-SDI×1 |
| 输入分辨率 | SDI - 12G-SDI/Dual-Link 6G-SDI/Quad-Link 3G-SDI - 4096x2160p@60/50fps - 3840x2160p@60/50fps - 6G-SDI - 4096x2160p@30/25/24fps - 3840x2160p@30/25/24fps - 3G-SDI - 1920×1080p@60/50fps - HD-SDI - 1920×1080p@30/25/24fps - 1920×1080i@60/50fps |

12G-SDI to HDMI2.0-S规格书

|  | - 1280×720p@60/50fps |  |
| --- | --- | --- |
| 输出分辨率 | - HDMI 2.0 ( Up/Down Converted ) - 4096x2160p@60/50fps - 4096x2160p@30/25/2 4fps - 3840x2160p@60/50fps - 3840x2160p@30/25/2 4fps - 1920×1080p@60/50fp s - 1920×1080p@30/25/2 4fps - 1920×1080i@60/50fps - 1280×720p@60/50fps - 720×480i@60fps - 720×576i@50fps | SDI ( Loop-Through ) - 12G-SDI - 4096x2160p@60/50fps - 3840x2160p@60/50fps - 6G-SDI - 4096x2160p@30/25/24fps - 3840x2160p@30/25/24fps - 3G-SDI - 1920×1080p@60/50fps - HD-SDI - 1920×1080p@30/25/24fps - 1920×1080i@60/50fps - 1280×720p@60/50fps |
| 音频输入 | SDI : Up to 8 Channels, 24bit, 48KHz |  |
| 音频输出 | HDMI : Up to 8 Channels, 24bit, 48KHz 3.5 mm Phone Jack : 2 Channels, 24bit, 48KHz |  |
| Plug & Play | ✓ |  |
| 电源 | 7.5V ~ 15V |  |
| 尺寸 | L:190mm×W:76mm×H:26mm |  |

2/3

| Connection Diagram | Single Link / Dual Link / Quad Link Connection For 12G-SDI or 6G-SDI, User can connect it on 12G SDI IN 1 or 12G SDI IN 3 |
| --- | --- |

3/3